Jitter control

Jitter control aims to manage and reduce variations in the timing of data packet delivery, crucial for applications like VoIP and video conferencing, which are sensitive to delays. Methods include using jitter buffers, upgrading network infrastructure, and minimizing bandwidth usage.

What is Jitter?

- Jitter refers to the variability or inconsistency in the timing of data packets arriving at their destination.

- It's a key factor affecting the quality of real-time applications, such as voice and video communications, where delays can be disruptive.

- Think of it as unpredictable delays in the arrival of data packets, similar to the delays when trying to connect to a network.

Causes of Jitter:

- Network Congestion:

High traffic on the network can lead to delays and variations in packet delivery times.

- Unpredictable Network Latency:

Fluctuations in the time it takes for data to travel across the network can cause jitter.

- Outdated Network Equipment:

Older network devices, such as routers, switches, or cables, might not be able to handle data traffic efficiently, leading to jitter.

- Unnecessary Bandwidth Usage:

High bandwidth consumption during work hours by streaming, downloading, or other non-critical applications can contribute to network congestion and jitter.

How to Reduce Jitter:

Jitter Buffers:

  - These act as temporary storage for incoming data packets.

  - They allow the application to receive packets at a more constant rate, smoothing out the variations in timing.

  - VoIP endpoints like desk phones often include jitter buffers to handle packet delays.

Upgrade Network Infrastructure:

  - Investing in high-quality network equipment like routers, switches, and cables can improve network performance and reduce jitter.

  - Using newer cables (e.g., 250 MHz cables) that can transmit data at higher speeds can help.

Minimize Bandwidth Usage:

  - Reduce unnecessary bandwidth consumption during working hours by avoiding high bandwidth activities like streaming or online gaming.

  - Schedule updates and backups for off-peak hours.

Use a Wired Connection:

  - A wired connection (Ethernet) is generally more reliable and stable than a wireless connection (Wi-Fi), reducing the likelihood of jitter.

Monitor Network Performance:

  - Monitor network traffic, latency, and bandwidth usage to identify and address potential issues before they impact real-time applications.

  - Use network monitoring and analysis tools to detect patterns and understand the root causes of jitter.

# Remote Procedure Call (RPC)

A remote procedure call is an interprocess communication technique that is used for client-server based applications. It is also known as a subroutine call or a function call.

The sequence of events in a remote procedure call are given as follows −

- The client stub is called by the client.

- The client stub makes a system call to send the message to the server and puts the parameters in the message.

- The message is sent from the client to the server by the client’s operating system.

- The message is passed to the server stub by the server operating system.

- The parameters are removed from the message by the server stub.

- Then, the server procedure is called by the server stub.

- A diagram that demonstrates this is as follows −

## Advantages of Remote Procedure Call

- Remote procedure calls support process oriented and thread oriented models.

- The internal message passing mechanism of RPC is hidden from the user.

- The effort to re-write and re-develop the code is minimum in remote procedure calls.

- Remote procedure calls can be used in distributed environment as well as the local environment.

- Many of the protocol layers are omitted by RPC to improve performance.

## Disadvantages of Remote Procedure Call

- The remote procedure call is a concept that can be implemented in different ways. It is not a standard.

- There is no flexibility in RPC for hardware architecture. It is only interaction based.

- There is an increase in costs because of remote procedure call.

# Real Time Transport Protocol (RTP):

A protocol is designed to handle real-time traffic (like audio and video) of the Internet, is known as Real Time Transport Protocol (RTP). RTP must be used with UDP. It does not have any delivery mechanism like multicasting or port numbers. RTP supports different formats of files like MPEG and MJPEG. It is very sensitive to packet delays and less sensitive to packet loss.

History of RTP : This protocol is developed by Internet Engineering Task Force (IETF) of four members:

RTP is first time published in 1996 and known as RFC 1889. And next it published in 2003 with name of RFC 3550.

Applications of RTP :

- RTP mainly helps in media mixing, sequencing and time-stamping.

- Voice over Internet Protocol (VoIP)

- Video Teleconferencing over Internet.

- Internet Audio and video streaming.

RTP Header Format : The diagram of header format of RTP packet is